Autoseis Hammer Shear Wave Generator

Model H-GRT-HT-001

For use in down-hole shear wave velocity measurement systems

Manufactured under license from Georgia Tech Research Corporation, USA

- Controlled, reproducible generation of horizontally polarized vertically propagating shear waves

- Energy of impact is transmitted to the ground parallel to the ground surface

- Less horizontal shear stress in the ground

- Manual or Automatic Operation

- Rugged, field tested design

For Sales and Customer Service:

Finite Designs, Inc.

125 Wes Walker Memorial Drive

Ball Ground, GA 30107 USA

Tel:		(678) 880-3935

Fax:		(678) 880-3936

Email:	sales@finiteprecision.com

Autoseis Hammer Shear Wave Generator

Model H-GRT-HT-001

The Autoseis Hammer Shear Wave Generator is a remotely operated device for generating horizontally polarized vertically propagating shear waves for use in down-hole shear wave velocity measurement systems.  The seismic signals are generated by an offset mass rotating in a plane perpendicular to the ground surface, which impacts an object coupled to the ground surface at the moment the motion of the mass is tangent to the ground surface.  The energy of impact is transmitted to the ground parallel to the ground surface.  Autoseis Hammer Shear Wave Generator uses a rotating motion instead of a horizontal motion.  Because the acceleration is not horizontal, the reaction forces caused by acceleration of the mass create less horizontal shear stress in the ground prior to the desired time (impact).

The shear wave is generated by a steel hammer striking a steel anvil.  The hammer is accelerated in a rotating manner causing it to strike an anvil with sufficient force to generate a repeatable shear wave that can be detected by a geophone implanted in the ground.

The most unique part of the Autoseis Hammer Shear Wave Generator is the rotation of the hammer through 90 degrees to a position parallel with the ground surface where it is released.  In this position, all reactions from the release of the hammer are in the vertical direction.  This is important because it permits the release and generation of a shear wave without the reaction impulse of linear actuated devices.

For questions or comments concerning application and theory, contact:

Paul W. Mayne, PhD, P.E.

Professor - Geosystems Engineering Group

School of Civil & Environmental Engineering

Georgia Institute of Technology

790 Atlantic Drive, Mason Bldg 241

Atlanta, GA 30332-0355

Tel:	404-894-6226

Fax:	404-894-2281

Email:	paul.mayne@ce.gatech.edu

For Sales, Customer Service, Delivery or Warranty inquiries, contact:

Finite Designs, Inc.

125 Wes Walker Memorial Drive

Ball Ground, GA 30107 USA

Tel:	(678) 880-3935

Fax:	(678) 880-3936

Email:	sales@finiteprecision.com

For information concerning the piezoelectric trigger, contact:

GISCO Geophysical Instrument & Supply Co.

6323 Cambridge Street

Minneapolis, MN 55416 USA

Tel:	(800) 523-1526

Fax:	(952) 926-5498

Email:	sales@giscogeo.com

Specifications:

Hammer and Control Box, Electrical

110/120 VAC

50/60 Hz

Control Box

Positive terminal (red) 12-VDC banana clip port

Negative terminal (black) 12-VDC banana clip port

Hammer Box, Piezoelectric Trigger

Type: Gisco G-ST-04-02-001-02

Output: Contact closure (open collector NPN)

Pulse Width:  5-ms

Max Voltage:  20-volts

Max Amperage:  100-ma

Protection:  Over current, over voltage and reverse polarity

Size:  1-inch diameter x 3.5-inch length

(25mm diameter x 88mm length)

H-GTR-HT-001-MSF-0 2007-05-10